Newland Auto-id opos

NLS OPOS  supports EM2028\HR200\FM100\FM420 communication with HID POS、USB-DataPipe、COM(HR100 only supports COM).

（Attention: If the device communicates with COM port emulation or USB-DataPipe, please install udp_and_vcom_drv(driver).）

runtime environment:

- Install http://www.monroecs.com/files/OposCCOs-1_13_001.exe first install it by default.  The 1.13 CCOs supports all OPOS APIs from Release 1.01 through Release 1.13. for more information about the opos ccos please refer to http://monroecs.com/oposccos_current.htm

- Install oposSOinstaller.msi by default, this is the Newland auto-id opos service object for Newland scanner.

- If Newland scanner use USB Datapipe or virtual COM port to communication, please install udp_and_vcom_drv.1.7.5.Setup_20100629.exe, this is the Newland Auto-id USB Datapipe and virtual COM port driver.

configure

Now ,you can use AdminTool.exe(Start -> All programs -> Newland Opos Scanner -> AdminTool) to setting Newland scanner's communication on PC. (Remark : application must “run as administrator” under the windows 7  if you want to change communication type)

Choose NLS_EM2028, and communication type choose HID, then click Save button to complete setting.

(note: Please refer to EM2028/HR200/FM420 Integration Guide ( http://www.newland-id.com/?p=3 ) to setting self-device's communication, if you use COM or USB-DataPipe.)

Operation

1.  EM2028\HR200\FM420 connect to PC by USB or RS232 cable(HR100 use RS232 cable).

2.  run .\NLSOposTest\Release\NLSOPOSTest.exe(or “Start -> All programs -> Newland Opos Scanner -> NLSOPOSTest”) to testing. (Remark : application must “run as administrator” under the windows 7; double-click to running under the windows XP).

3. Device name fill-in:

“NLS_EM2028”  “NLS_HR200”  “NLS_FM420”  “NLS_FM100”

“Open”->”Claim”->”Device Enable” to scan.

Test by website

Fill-in “NLS_EM2028” to Device name

- EM2028\HR200\FM420 connect to PC by USB or RS232 cable(HR100 use RS232 cable).

- Open  http://www2.clearlake.ibm.com/store/support/tools/scannerexample.html   and Device Name fill-in “NLS_EM2028”.

- Click "Open/Claim/Enable",  “Results” will display  “Open() = 0....”.